Configuring Microsoft Flight Simulator 2000 Controls

by Pete Dowson, CIS 100041,312 (pete@millhays.demon.co.uk)

2nd Edition: 24th November 1999

INTRODUCTION

This document is mainly just a list of the controls available in Flight Simulator 2000 (FS2K) and how they can be assigned to keyboard key-presses and/or joystick axes and buttons. It does not try to teach anyone to use the ‘Controls Assignments’ dialogue in FS2K – the features listed cannot all be assigned in that way in any case.

There are many more controls available than those assigned by default. You can have keyboard or cockpit control over anything you need to adjust or change whilst flying.

At this time this is only a quick revision of my FS98 document to list the changes from FS98 to FS2K, and the extra controls. I have not had time to test all the new controls, nor even to verify that all the old ones work as they used to—from what I’ve seen so far, it is highly unlikely that all of them will work in the same way. Try what you need and see. If you find anything out that I’ve not mentioned, please let me know. I’ll collate all the information I get for a 3rd edition.

CHANGES IN THIS EDITION

The ‘SPOILERS_SET’, ‘MIXTURE_SET’ and ‘PROP_PITCH_SET’ controls are still available as they were in FS98. The new ‘AXIS_…_SET’ versions appear to be additional not replacements, and operate using the full range of the axis, not just half as (at least) the old SPOILERS_SET control does.

The control ‘VIEW_RESET’ was omitted from the list.

FS Button 40 appears to be available, activating on a “POV off” input – eg when a POV hat is released so it springs to centre. In the cases observed it is allocated to the VIEW_RESET control to give the momemtary action on the view selection, as an alternative to the BUTTON_UP assignments in most cases.

EDITING the FS2000.CFG file

Before making any changes to your FS2000.CFG file, make a copy. Alternatively, make a copy for your experiments, called say “TEST.CFG” and keep it in the FS98 folder. Make changes to this rather than the original. You can run FS2K with this alternative CFG file by running it with the command line:

FS2000.EXE /CFG:TEST.CFG

To make an FS2K icon to run this, just make a copy of the current FS2K icon (hold CTRL down and drag a copy somewhere), then right-click the Icon, select ‘properties’ and add the /CFG:TEST.CFG bit to the Shortcut “target” string you will find there (add this inside the quotation marks, with a space separating the addition from the original string). Don’t forget to rename the FS2K icon to something suitable (right-click the icon and select ‘Rename’).

I am only going to discuss two areas of the CFG file – the [KEYBOARD ...] and [JOYSTICK ...] sections. No doubt the other sections are of great interest also, but the scope of this document is restricted to ‘controls’ that can be allocated to keyboard and joysticks – both how to do this, and, as far as is understood, what the controls are.

The latter will expand as more information is collected, more tests are performed, and in particular, what questions are asked. Many of the controls listed in the Appendix are, to me, self evident, so I find them difficult to explain any differently. So there are just a few comments where I thought there might be confusion or ambiguity. It is up to you, the readers, to ask questions where you don’t understand, and I will attempt to improve this document, time permitting, in response.

IMPORTANT

The very first thing to do, if you are an FS98 user moving on to FS2K, is to make FS2000 obey your joystick settings properly, using exactly the same calibrations and assignments as you had in FS98. To do this, add the following line to the [CONTROLS] section of FS2000.CFG:

stick_sensitivity_mode=0

Now you can copy and paste your entire set of  [JOYSTICK … sections from your FLTSIM98.CFG file into your FS2000.CFG file, replacing any already generated there. Without this new line, FS2K does something pretty screwy with the Joystick calibrations, and, even worse, really fouls up the POV inputs (AXIS_06 settings, described later).

If you want the same Keyboard Controls as in FS98 you can copy over the [KEYBOARD_ …] sections too, but if you used the [KEYBOARD_EXTRA] section in FS98 (to prevent FS98 removing those controls added after the initial release), you’ll need to move all the settings from that section into the [KEYBOARD_MAIN] section. Don’t worry. It seems FS2K is not as nasty as FS98 here—it won’t delete all your hard work! (But make a backup just in case! <G>).

Keyboard Controls

There are two keyboard configurations:

[KEYBOARD_MAIN] and [KEYBOARD_SLEW]

The MAIN section is used until SLEW mode is entered (press Y), then the SLEW section is used. Most of the controls you’ll want to configure will be in the MAIN section. Not very many are truly useful in Slew mode.

Keyboard controls are assigned in these sections by a line for each. For example:

PANEL_1=49,9

PANEL_2=50,9

PANEL_3=51,9

These three lines define the three key combinations which are assigned to the three controls ‘PANEL_1’, ‘PANEL_2’ and ‘PANEL_3’, These controls toggle the first three panels on or off – panel 1 is the main panel, the others vary according to aircraft.

The numbers on the right represent the key presses needed in two parts:

49,9	means ‘keycode’ 49, and ‘shift-state’ 9.

Appendix 1 gives a list of all these. in this case you will see that keycode 49 refers to the main keyboard ‘1’ (number 1) key, whilst the shift-state value 9 indicates that a Shift key must also be pressed.

See Appendix 2 for the list of control names to which you can assign keypresses. Note that there are more controls available that those which are listed in FS2K’s “Options–Controls—Assignments” dialogue.

Joystick Controls

The joystick control in FS2K is contained in the [JOYSTICK_xx] sections in the FS2000.CFG file, found in the FS98 main directory. “xx” is 00 for the 1st Joystick, up to 15 for the 16th Joystick (the maximum number supported).

As usual, before changing anything, make a copy of FS2000.CFG somewhere for safe keeping.

In FS2K each JOYSTICK actually has two sections:

[JOYSTICK_nn]   and  [JOYSTICK_SLEW_nn]

with nn running from 00 to 15, assuming you installed all 16 possible joysticks.

I suspect, like FS98 before it, FS2K processes the CFG file and often re-writes it. Unless we take precautions, it may seem to do strange things, like giving us multiple joysticks all apparently configured to control the same things. Also, unless we take precautions, it may overwrite all the careful work we might do with its own view of how things should be.

If you are not copying over all your JOYSTICK settings from FS98, as suggested earlier, the first step is to put “LOCKED=1” in every [JOYSTICK ...] section. This will stop it being changed so easily. It should also stop you changing it from within FS2K, but it will be worth it.

### Analogue Axes

The analogue inputs are enabled by:

- Adding a ‘bit’ into the AXIS_FLAGS for that input. Add values together, as follows:

- 1 for Axis 0 (the X axis, typically aileron)

- 2 for Axis 1 (the Y axis, typically elevator)

- 4 for Axis 2 (the Z axis, typically throttle)

- 8 for Axis 3 (the R axis, typically rudder)

- 16 for Axis 4 (the U axis)

- 32 for Axis 5 (the V axis)

- 64 for Axis 6 (this is actually the “POV”, Point Of View hat)

- So, for example 15 indicates the usual basic 4 joystick axes (X,Y,Z,R).

- Adding 3 lines per Axis:

- AXIS_EVENT_nn=<control name>

- AXIS_SCALE_nn=<scale>

- AXIS_NULL_nn=<null>

- where:

- nn is the Axis number (00–06)

- <control name> is simply that: select a suitable analogue control from the list!

- <scale> controls sensitivity. For any well calibrated joystick, the value 64 is usually the best start here for all normal axes, although 128 for full throttle (THROTTLE_SET) works well. A negative number here reverses the sense of the control, making it increment with the opposite movement.

- Note: for discrete value axis controls, listed later, the SCALE value needs to be 1 or –1 (although the negative value won’t find much use). Such controls need to be allocated only to Axis 6 in each Joystick section, as the other inputs will be affected by calibration.

- <null> is best set 0 for throttles and analogues other than the three primary flying controls, unless the analogue axis concerted has any “jitter” (i.e. is not stable at centre or either extreme, wherever the ‘normal’ setting is). The null zone on the main flying controls is a matter for personal preference—for helicopter flying I’d advise a very small amount!

- Note: Always set a zero value for the NULL zone for discrete value axis controls, as described later.

Note that, apart from the convention already adopted, and for compatibility, there is no specific requirement to associate throttles with Axis 2 and ailerons with Axis 0, and so on. However, you should note that the “centre” calibration system, as appropriate to aileron, elevator and rudder, is only applied to X, Y and R axes. Where centering is not important, as in most all other cases, this allocation is not really so relevant.

### Buttons

The button assignment lines are simpler. Each joystick section can have assignments for buttons 00–40. These represent joystick buttons 0–31, plus the 8 directions and no direction on an 8-way “Point of View” (POV) control (32–39 for the 8 directions, 40 for ‘off’). There is no specific need to use points of view type controls with the POV inputs. They can be used for any of the button controls.

Each used button has one or maybe two lines:

BUTTON_DOWN_EVENT_nn=<control name>

BUTTON_DOWN_REPEAT_nn=<rate>

and even (new to FS2K):

BUTTON_UP_EVENT_nn=<control name>

where:

nn is the button number (00–40)

<control name> is simply that: select a suitable non-analogue control from the list, later

<rate> is 1 for fastest repeat (every frame). Higher numbers give lower repeat rates.

The REPEAT line is optional, and, in fact, seldom used. The main use here is probably for panning (with the POVs) and braking. Remember that the repeat rate is completely dependent on the current frame rate.

The BUTTON_UP event setting allows the new default view actions to be implemented—holding the button down keeps the selected view and releasing it restores the forward view. For the old type of view changing option, just don’t use the BUTTON_UP_EVENT entries.

### Brief survey of the FS2000 controls

Some specific clarification of some of the controls is included here, to pass on the benefit of the experiments and results I’ve had. There are many more controls to try so please go ahead. I’d be interested to know of other user’s findings. I’ll add more information in here as needed.

Autopilot controls: There are often ‘HOLD’ controls and separate ‘HOLD_ON’, ‘HOLD_OFF’ controls. You won’t need to use all three. For keyboard allocations and push buttons the toggle on/off action of the simple ‘HOLD’ will do. For panel switches which visibly show their position as on or off (toggle and aircraft-type electric switches), you are better off using the specific HOLD_ON and HOLD_OFF controls, otherwise your panel can get ‘out of sync’ with the simulator. (A similar consideration applies to some other controls which have a “TOGGLE” and also separate “ON” and “OFF” variations).

The ‘AP_PANEL_xxx’ controls are very useful. They switch the AP hold action using the value pre-set into the AP dial (using the VAR_INC and VAR_DEC controls). You will find both types of HOLD available for all of the dialled values, and there are (now) ON and OFF controls for all of these too:

AP_PANEL_ALTITUDE_HOLD		AP_ALT_HOLD

AP_PANEL_HEADING_HOLD		AP_HDG_HOLD

AP_PANEL_MACH_HOLD		AP_MACH_HOLD

AP_PANEL_SPEED_HOLD		AP_AIRSPEED_HOLD

AP_PANEL_VS_HOLD			AP_VS_HOLD

Note that the Panel MACH and SPEED share a dial. They switch over (when flying) by use of the appropriate HOLD control.

There are some new ones in this category which I haven’t yet investigated:

AP_PANEL_CRUISE_SPEED_TOGGLE

AP_PANEL_MACH_HOLD_TOGGLE

AP_PANEL_MAX_SPEED_TOGGLE

AP_PANEL_SPEED_ACQ_TOGGLE

AP_PANEL_SPEED_HOLD_TOGGLE

Of these the two ‘HOLD_TOGGLE’s seem to duplicate existing controls (the shorter-named _HOLD ones which do toggle anyway), but I’ve not yet tested them to see if there are any differences.

Other notes:

The interestingly named “KOHLSMAN” (a mis-spelling of “Kollsman”, a famous maker of aircraft instruments in the early days of aviation) controls change the altimeter’s barometric pressure reading, whilst  the “BAROMETRIC” restores this to the prevailing pressure (QNH).

The elevator trim can be controlled by an analogue input (AXIS_ELEV_TRIM_SET) but otherwise is (oddly) controlled only by NUMPAD1 and NUMPAD7. (In FS2K there are new controls for AILERON trim and RUDDER trim, but the important elevator trim is still called NUMPAD1 and NUMPAD7 after the default key assignments.)

The ANTI_ICE_TOGGLE operates the Carb Heat in the Cessna (and other props I expect). It may actually work the anti-icing in jets in FS2K—but there are new “deicing” controls in the “TOGGLE_” range (see the list in the Appendix).

I still don’t really know what JOYSTICK_CALIBRATE does, but it seems likely to do the same as the ‘K’ key—i.e. switch joystick control off. I wouldn’t recommend this for joystick button assignment, as it is an effective suicidal gesture!

PANEL_1 through to PANEL_9 toggle specific panel parts and these will depend on how the current panel is configured. Their use is actually defined in text files in the aircraft directories, so you can always change things over to suit your controls. There are also VIEW_AUX_nn controls, new to FS2K, which may do similar things.

THROTTLE1_SET to THROTTLE4_SET operate rather differently to the AXIS_THROTTLE_SET control. To obtain reverse thrust without using these four separate throttle inputs you have to set the AXIS_THROTTLE_SET input to off (255) and resort to the THROTTLE_CUT then THROTTLE_DECR button controls.

There are three new Axis controls to be aware of:

AXIS_MIXTURE_SET		as well as MIXTURE_SET (as in FS98)

AXIS_PROPELLER_SET	as well as PROP_PITCH_SET (as in FS98)

AXIS_SPOILER_SET 		as well as SPOILERS_SET (as in FS98)

I think these new Axis controls all act on the full range of the joystick, whereas certainly the SPOILERS_SET, and possibly the other two older ones also, only operate on half of the range.

Finally, please note that in FS98 I found some cases where controls labelled as “SMALL” (as, for example, in PROP_PITCH_INCR_SMALL) were actually the correct equals to their “non-SMALL” opposites (DECR), whilst the ones not labelled SMALL were a lot smaller. I think this oddity also applied to FSW95, and it may or may not have been continued in FS2K.

Oh, yes, and take care to get the spelling of FS2K controls exactly as they are in the list. Like FS98, FS2K ruthlessly deletes any that are wrong, without a murmur! It is easy to make mistakes, especially as some of the abbreviations are inconsistent, such as DEC or DECR, INC or INCR.

APPENDIX 1:  List of Keyboard Codes

The keyboard sections of FS2000.CFG use Windows 'virtual keycodes' to define the keypresses being assigned. Two numbers are given, separated by a comma. The first is the virtual keycode from the list below. The second is a "shift state" indicator, thus:

8	plain key only

9	SHIFT + key

10	CNTRL + key

11	SHIFT + CNTRL + key

There are also values 40, 41, 42 and 43—as above with 32 added. I don’t know what the 32 addition does. Experiments here seem to indicate that it makes no difference—they act the same as if the 32 hadn’t been added. (If anyone finds out, please let me know!)

Here is the list of keycodes, in code number order. NOTE that these may not all be usable in FS2K. Also, those marked with an asterisk (*) are often keyboard specific. The values shown are for the UK keyboard. Please see the text for how to identify keycodes for specific keys on your keyboard.

3	Ctrl Break (needs shift state = 10)

8	Backspace

9	Tab

12	NumPad 5 (NumLock OFF)

13	Enter

19	Pause

20	Caps lock (usable with Shift but not with Cntrl)

27	Escape

32	Space bar

33	Page Up

34	Page Down

35	End

36	Home

37	Left arrow

38	Up arrow

39	Right arrow

40	Down arrow

44	Print screen (not usable)

45	Insert

46	Delete

48	0 on main keyboard

49	1 on main keyboard

50	2 on main keyboard

51	3 on main keyboard

52	4 on main keyboard

53	5 on main keyboard

54	6 on main keyboard

55	7 on main keyboard

56	8 on main keyboard

57	9 on main keyboard

65	A

66	B

67	C

68	D

69	E

70	F

71	G

72	H

73	I

74	J

75	K

76	L

77	M

78	N

79	O

80	P

81	Q

82	R

83	S

84	T

85	U

86	V

87	W

88	X

89	Y

90	Z

96	NumPad 0 (NumLock ON)

97	NumPad 1 (NumLock ON)

98	NumPad 2 (NumLock ON)

99	NumPad 3 (NumLock ON)

100	NumPad 4 (NumLock ON)

101	NumPad 5 (NumLock ON)

102	NumPad 6 (NumLock ON)

103	NumPad 7 (NumLock ON)

104	NumPad 8 (NumLock ON)

105	NumPad 9 (NumLock ON)

106	NumPad *

107	NumPad +

109	NumPad -

110	NumPad .

111	NumPad /

112	F1

113	F2

114	F3

115	F4

116	F5

117	F6

118	F7

119	F8

120	F9

121	F10

122	F11

123	F12

135	NumPad Enter

144	NumLock

145	ScrollLock

186	; : Key*

187	= + Key*

188	, < Key*

189	- _ Key*

190	. > Key*

191	/ ? Key*

192	# ~ Key*

219	[ { Key*

220	\ | Key*

221	] } Key*

222	' @ Key*

223	` ¬ ¦ Key*

* These keys will vary from keyboard to keyboard. The graphics indicated are those shown on my UK keyboard. It is possible that keys in the same relative position on the keyboard will respond similarly, so here is a positional description for those of you without UK keyboards. This list is in left-to-right, top down order, scanning the keyboard:

223 	` ¬ ¦	is top left, just left of the main keyboard 1 key

189	- _	is also in the top row, just to the right of the 0 key

187	= +	is to the right of 189

219	[{	is in the 2nd row down, to the right of the alpha keys.

221	]}	is to the right of 219

186	; :	is in the 3rd row down, to the right of the alpha keys.

222	' @	is to the right of 186

192	# ~	is to the right of 222 (tucked in with the Enter key)

220	\ |	is in the 4th row down, to the left of all the alpha keys

188	, <	is also in the 4th row down, to the right of the alpha keys

190	. >	is to the right of 188

191	/ ?	is to the right of 190

APPENDIX 2:  List of Flight Simulator 2000 Controls

These are the control names which are used in FS2000.CFG. The names can be assigned to buttons and, in some cases, to axes, in the [JOYSTICK ...] sections. Keystrokes can be assigned to these names in the [KEYBOARD_MAIN] and [KEYBOARD_SLEW] sections. Remember, joysticks have controls assigned to them, whilst the keyboard assignments are to the control instead. (Crazy, eh?).

Numerical values are given as well for those controls which were also in FS98, but not for any of the new ones. The names should be used in FS2000. (Some of the numerical values will also apply to FLTSIM95—but not all of the controls can be used in FLTSIM95. Thanks are due to Bob Church for original information on the numeric equivalents.)

The list is in three parts: “discrete value axes”, “analogue axes” and “buttons”. Button controls are those which are either active (‘pressed’ or ‘on’) or inactive (not pressed, or ‘off’), and are operated by real buttons or by key presses. Axis controls are normally responsive to analogue values instead (the discrete value ones need special hardware rather than a normal joystick range).

Notes are added on the right where there might be some confusion or obscurity and I’ve got any helpful information. I’ll try to add to these in later editions. Those marked [*] are new to FS2000.

Analogue axis controls

AXIS_AILERONS_SET	65763	Normal aileron control

AXIS_ELEV_TRIM_SET	65766

AXIS_ELEVATOR_SET	65762	Normal elevator control

AXIS_MIXTURE_SET		 [*]	All engines unless changed by ENGINE selection (see also MIXTURE_SET)

AXIS_PROPELLER_SET		 [*]	All engines unless changed by ENGINE selection (see PROP_PITCH_SET)

AXIS_RUDDER_SET	65764	Rudder control from pedals

AXIS_SLEW_AHEAD_SET	65867

AXIS_SLEW_ALT_SET	65870

AXIS_SLEW_BANK_SET	65871

AXIS_SLEW_HEADING_SET	65869

AXIS_SLEW_PITCH_SET	65872

AXIS_SLEW_SIDEWAYS_SET	65868

AXIS_SPOILER_SET		 [*]	(see also SPOILERS_SET)

AXIS_THROTTLE_SET	65765	Normal throttle control

MIXTURE_SET	65773	All engines unless changed by ENGINE selection

MIXTURE1_SET	65919

MIXTURE2_SET	65920

MIXTURE3_SET	65921

MIXTURE4_SET	65922

PROP_PITCH_SET	65767	All engines unless changed by ENGINE selection

PROP_PITCH1_SET	65923

PROP_PITCH2_SET	65924

PROP_PITCH3_SET	65925

PROP_PITCH4_SET	65926

SPOILERS_SET	65786

THROTTLE1_SET	65820	Individual throttle controls

THROTTLE2_SET	65821

THROTTLE3_SET	65822

THROTTLE4_SET	65823

Discrete value axis controls

These controls are operated by specific analogue values. Information on these is given where known. They can only be successfully used on Axis 6 (POV) assignments, as the normal 6 analogue axes on each joystick will have original (“raw”) values transformed by Win95’s calibration system. You must have the line

stick_sensitivity_mode=0

in the [CONTROLS] section to use POV controls in this manner. Also the NULL_ZONE for these must be set zero, and the SCALE parameter to 1 (normally), or –1 (unlikely: reverses the value!).

ADF_CARD_SET	66039	0–359 degrees, sets non-slaved ADF card orientation

ADF_EXTENDED_SET		 [*]	Not yet investigated

ADF_HIGHRANGE_SET	66123 	BCD, 0x0000 to 0x6999 for ADF values 1000.0–1699.9

ADF_LOWRANGE_SET	66122 	BCD, 0x2000 to 0x9999 for ADF values 0200.0–0999.9

ADF_SET	65714 	BCD, 0x0200 to 0x0999. No fractions or high range in this one

ANTI_ICE_SET	66031 	Carb heat off (0) or on (1)

AP_AIRSPEED_SET	66095 	0=off, 1=on (switches hold on current value)

AP_ALT_VAR_SET_ENGLISH	66124 	Altitude in feet (0-65535)

AP_ALT_VAR_SET_METRIC	66035 	Altitude in metres

AP_MACH_SET	66098 	0=off, 1=on (switches hold on current value)

AP_MACH_VAR_SET	66038 	1/100ths Mach, so 100 = Mach 1.00

AP_PANEL_ALTITUDE_SET	66105 	0=off, 1=on (switches hold on panel-set value)

AP_PANEL_HEADING_SET	66108 	0=off, 1=on (switches hold on panel-set value)

AP_PANEL_MACH_SET	66111 	0=off, 1=on (switches hold on panel-set value)

AP_PANEL_SPEED_SET	66114 	0=off, 1=on (switches hold on panel-set value)

AP_PANEL_VS_SET	66117 	0=off, 1=on (switches hold on panel-set value)

AP_SPD_VAR_SET	66037 	0-65535 knots

AP_VS_SET	66102 	0=off, 1=on (switches hold on current value)

AP_VS_VAR_SET_ENGLISH	66036 	-7000 to +7000 ft/min

AP_VS_VAR_SET_METRIC	66125 	similar but in metres/min

AUTOCOORD_SET	65954 	1=on, 0=off

AXIS_IND_SET	65702 	0=off, 1=largeV, 2=4dots, 3=smallV

CENTERING_AILERON_SET		 [*]

CENTERING_ELEVATOR_SET		 [*]

CENTERING_RUDDER_SET		 [*]

CLOCK_HOURS_SET	66044 	0–23

CLOCK_MINUTES_SET	66045 	0–59

COM_RADIO_SET	65707 	BCD with 100 place assumed. e.g. 0x2485 for 124.85

EGT_SET	66034 	EGT bug setting 0–16383 = 0–100%, for selected Engines

EGT1_SET	66083 	EGT1 bug setting 0–16383 = 0–100%

EGT2_SET	66086 	EGT2 bug setting 0–16383 = 0–100%

EGT3_SET	66089 	EGT3 bug setting 0–16383 = 0–100%

EGT4_SET	66092 	EGT4 bug setting 0–16383 = 0–100%

FLAPS_DETENTS_SET	65760	Sets the number of flap positions. These are evenly spaced!

FLAPS_SET	65698 	Flap setting 0 none–16383 full

FUEL_SELECTOR_SET	65962 	0=off, 1=all, 2=left, 3=right, 4=Left aux, 5=Right aux, 6=Centre

GEAR_SET	65700 	0=up, 1=down

GYRO_DRIFT_SET	66046 	0-359 degrees

HEADING_BUG_SET	66042 	0–359 degrees

KOHLSMAN_SET	66040 	Sets altimeter pressure, in millibars x 16 (range 948 x 16 to 1084 x 16)

LANDING_LIGHTS_SET	66061	0=off, 1=on

MAGNETO_SET	66028 	Selected engines: 0-4 as below

MAGNETO1_SET	65932 	0=off, 1=right, 2=left, 3=both, 4=start; Jets: 0=off, 1=start, 2=gen

MAGNETO2_SET	65938 	as above

MAGNETO3_SET	65944 	as above

MAGNETO4_SET	65950 	as above

NAV1_RADIO_SET	65708 	BCD with 100 place assumed. e.g. 0x1050 for 110.50

NAV2_RADIO_SET	65709 	BCD with 100 place assumed. e.g. 0x1050 for 110.50

PANEL_LIGHTS_SET	66058 	0=off, 1=on

PAUSE_SET	66055 	0=off, 1=on

PITOT_HEAD_SET	66074 	0=off, 1=on

RADIO_ADF_IDENT_SET	65851	0=off, 1=on

RADIO_DME1_IDENT_SET	65849	0=off, 1=on

RADIO_DME2_IDENT_SET	65850	0=off, 1=on

RADIO_VOR1_IDENT_SET	65847	0=off, 1=on

RADIO_VOR2_IDENT_SET	65848	0=off, 1=on

SEE_OWN_AC_SET	66121 	0=see own aircraft off, 1=see own aircraft on

SIM_RATE_SET	66041 	Sets sim rate, 8:8 integer:fraction, range 0x0040–0x8000 (1/4 to 128x)

SLEW_SET	66048 	0=off, 1=on

SMOKE_SET	66051 	0=off, 1=on

SOUND_SET	65692 	0=off, 1=on

SPOILERS_ARM_SET	66068 	0=off, 1=on

STROBES_SET	66054 	0=off, 1=on

TRUE_AIRSPEED_CAL_SET	66043 	0–90 degrees

VIEW_TYPE_SET	65831	0=main, 1=map

VOR1_SET	65716 	Sets OBS1, 0–359

VOR2_SET	65717 	Sets OBS2, 0–359

XPNDR_SET	65715 	BCD 0x0000 to 0x7777

YAW_DAMPER_SET	66071 	0=off, 1=on

ZULU_DAY_SET	66077 	Zulu time day of year, 0–365/366

ZULU_HOURS_SET	66075 	Zulu time of day, 0–23 hrs

ZULU_MINUTES_SET	66076 	Zulu time of day, 0–59 mins

ZULU_YEAR_SET	66078 	Zulu time, year. E.g. 1997

Button controls

ABORT	65733

ADF	65566

ADF_1_DEC	65668

ADF_1_INC	65650

ADF_10_DEC	65667

ADF_10_INC	65649

ADF_100_DEC	65666

ADF_100_INC	65648

ADF_CARD_DEC	65882

ADF_CARD_INC	65881

AILERON_TRIM_LEFT		 [*]

AILERON_TRIM_RIGHT		 [*]

AI_STUB_0		 [*]

AI_STUB_1		 [*]

AI_STUB_2		 [*]

AI_STUB_3		 [*]

AI_STUB_4		 [*]

AI_STUB_5		 [*]

AI_STUB_6		 [*]

AI_STUB_7		 [*]

AI_STUB_8		 [*]

AI_STUB_9		 [*]

AI_STUB_A		 [*]

AI_STUB_B		 [*]

AI_STUB_C		 [*]

AI_STUB_D		 [*]

AI_STUB_E		 [*]

AI_STUB_F		 [*]

AI_STUB_G		 [*]

AI_STUB_H		 [*]

AI_STUB_I		 [*]

AI_STUB_J		 [*]

AI_STUB_K		 [*]

AI_STUB_L		 [*]

AI_STUB_M		 [*]

AI_STUB_N		 [*]

AI_STUB_O		 [*]

AI_STUB_P		 [*]

AI_STUB_Q		 [*]

AI_STUB_R		 [*]

AI_STUB_S		 [*]

AI_STUB_T		 [*]

AI_STUB_U		 [*]

AI_STUB_V		 [*]

AI_STUB_W		 [*]

AI_STUB_X		 [*]

AI_STUB_Y		 [*]

AI_STUB_Z		 [*]

ALL_LIGHTS_TOGGLE	65574

ANALYSIS_MANEUVER_STOP	65579

ANTI_ICE_OFF	66030 	Carb heat off

ANTI_ICE_ON	66029 	Carb heat on

ANTI_ICE_TOGGLE	65571	Operates Carb Heat. No action on jets.

AP_AIRSPEED_HOLD	65859	Toggles current airspeed hold

AP_AIRSPEED_OFF	66094

AP_AIRSPEED_ON	66093

AP_ALT_HOLD	65726	Toggles current altitude hold

AP_ALT_HOLD_OFF	65816

AP_ALT_HOLD_ON	65808

AP_ALT_VAR_DEC	65893	Decrements AP panel altitude value

AP_ALT_VAR_INC	65892	Increments AP panel altitude value

AP_APR_HOLD	65724

AP_APR_HOLD_OFF	65814

AP_APR_HOLD_ON	65806

AP_ATT_HOLD	65722

AP_ATT_HOLD_OFF	65812

AP_ATT_HOLD_ON	65804

AP_BC_HOLD	65728

AP_BC_HOLD_OFF	65818

AP_BC_HOLD_ON	65810

AP_HDG_HOLD	65725	Toggles current heading hold

AP_HDG_HOLD_OFF	65815

AP_HDG_HOLD_ON	65807

AP_LOC_HOLD	65723

AP_LOC_HOLD_OFF	65813

AP_LOC_HOLD_ON	65805

AP_MACH_HOLD	65918	Toggles current Mach speed hold

AP_MACH_OFF	66097

AP_MACH_ON	66096

AP_MACH_VAR_DEC	65917	Decrements AP panel Mach value

AP_MACH_VAR_INC	65916	Increments AP panel Mach value

AP_MASTER	65580	Toggles AP on/off—see AUTOPILOT_ON/OFF

AP_MASTER_ALT	65669	Toggles AP on/off—see AUTOPILOT_ON/OFF

AP_NAV1_HOLD	65729

AP_NAV1_HOLD_OFF	65819

AP_NAV1_HOLD_ON	65811

AP_PANEL_ALTITUDE_HOLD	65799	Toggles preset panel altitude hold

AP_PANEL_ALTITUDE_OFF	66104

AP_PANEL_ALTITUDE_ON	66103

AP_PANEL_CRUISE_SPEED_TOGGLE	 [*]

AP_PANEL_HEADING_HOLD	65798	Toggles preset panel heading hold

AP_PANEL_HEADING_OFF	66107

AP_PANEL_HEADING_ON	66106

AP_PANEL_MACH_HOLD	65915	Toggles preset panel Mach hold

AP_PANEL_MACH_HOLD_TOGGLE		 [*]	?

AP_PANEL_MACH_OFF	66110

AP_PANEL_MACH_ON	66109

AP_PANEL_MAX_SPEED_TOGGLE		 [*]

AP_PANEL_SPEED_ACQ_TOGGLE		 [*]

AP_PANEL_SPEED_HOLD	65891	Toggles preset panel airspeed hold

AP_PANEL_SPEED_HOLD_TOGGLE		 [*]	?

AP_PANEL_SPEED_OFF	66113

AP_PANEL_SPEED_ON	66112

AP_PANEL_VS_HOLD	65890	Toggles preset panel vertical speed hold

AP_PANEL_VS_OFF	66116

AP_PANEL_VS_ON	66115

AP_SPD_VAR_DEC	65897	Decrements AP panel airspeed

AP_SPD_VAR_INC	65896	Increments AP panel airspeed

AP_VS_HOLD	66099 	Toggles current vertical speed hold

AP_VS_OFF	66101

AP_VS_ON	66100

AP_VS_VAR_DEC	65895	Decrements AP panel vertical speed

AP_VS_VAR_INC	65894	Increments AP panel vertical speed

AP_WING_LEVELER	65727

AP_WING_LEVELER_OFF	65817

AP_WING_LEVELER_ON	65809

ATC	65564

ATTITUDE_BARS_POSITION_DOWN	 [*]	Lowers the little aircraft position in the AI

ATTITUDE_BARS_POSITION_UP		 [*]	Raises the little aircraft position in the AI

ATTITUDE_CAGE_BUTTON		 [*]

AUTO_THROTTLE_ARM	65860

AUTO_THROTTLE_TO_GA	65861

AUTOCOORD_OFF	65952

AUTOCOORD_ON	65953

AUTOCOORD_TOGGLE	65951

AUTOPILOT_OFF	65791

AUTOPILOT_ON	65792

BAROMETRIC	65584	Resets altimeter barometer setting to QNH

BRAKES	65588

BRAKES_LEFT	65720

BRAKES_RIGHT	65721

CENTER_NT361_CHECK	65803

CHECKLIST_VIEW	65876

CHVPP_AP_ALT_WING	65802

CHVPP_LEFT_HAT_DOWN	65801

CHVPP_LEFT_HAT_UP	65800

CLOCK_HOURS_DEC	65657

CLOCK_HOURS_INC	65658

CLOCK_MINUTES_DEC	65659

CLOCK_MINUTES_INC	65660

CLOCK_SECONDS_ZERO	65661	NOT a reset for the timer, only the clock seconds

CLOSE_VIEW	65824

COM_RADIO	65582

COM_RADIO_FRACT_DEC	65638

COM_RADIO_FRACT_INC	65639

COM_RADIO_WHOLE_DEC	65636

COM_RADIO_WHOLE_INC	65637

COM_STBY_RADIO_FRACT_DECREASE	 [*]

COM_STBY_RADIO_FRACT_INCREASE	 [*]

COM_STBY_RADIO_SWITCH_TO	 	 [*]

COM_STBY_RADIO_WHOLE_DECREASE	 [*]

COM_STBY_RADIO_WHOLE_INCREASE	 [*]

CROSS_FEED_LEFT_TO_RIGHT	65888

CROSS_FEED_OFF	65887

CROSS_FEED_RIGHT_TO_LEFT	65889

DECREASE_AUTOBRAKE_CONTROL	 [*]

DECREASE_CONCORDE_NOSE_VISOR	 [*]

DECREASE_CONCORDE_NOSE_VISOR_FULLY [*]

DECREASE_DECISION_HEIGHT		 [*]

DECREASE_HELO_GOV_BEEP		 [*]

DEC_COWL_FLAPS		 [*]

DEC_COWL_FLAPS1		 [*]

DEC_COWL_FLAPS2		 [*]

DEC_COWL_FLAPS3		 [*]

DEC_COWL_FLAPS4		 [*]

DEMO_RECORD_1_SEC	65543

DEMO_RECORD_5_SEC	65544

DEMO_RECORD_MESSAGE	65587

DEMO_STOP	65537

DME	65569

DME1_TOGGLE	65787

DME2_TOGGLE	65788

EGT	65558	Selects EGT needle, use PLUS/MINUS to move it

EGT_DEC	66033 	Operates on selected engine EGT(s)

EGT_INC	66032 	Operates on selected engine EGT(s)

EGT1_DEC	66082

EGT1_INC	66081

EGT2_DEC	66085

EGT2_INC	66084

EGT3_DEC	66088

EGT3_INC	66087

EGT4_DEC	66091

EGT4_INC	66090

ENGINE	65554	Use SELECT_n to select engines for operations

ENGINE_AUTO_START		 [*]

EXIT	65732

End		 [*]	?

FLAPS_1	65597

FLAPS_2	65599

FLAPS_3	65601

FLAPS_DECR	65759

FLAPS_DOWN	65603

FLAPS_INCR	65758

FLAPS_UP	65595

Force		 [*]	?

FUEL_SELECTOR_ALL	65956

FUEL_SELECTOR_CENTER	65961

FUEL_SELECTOR_LEFT	65957

FUEL_SELECTOR_LEFT_AUX	65959

FUEL_SELECTOR_OFF	65955

FUEL_SELECTOR_RIGHT	65958

FUEL_SELECTOR_RIGHT_AUX	65960

FULL_WINDOW_TOGGLE	65553

GEAR_DOWN	66080

GEAR_PUMP	65852

GEAR_TOGGLE	65570

GEAR_UP	66079

GYRO_DRIFT_DEC	65878

GYRO_DRIFT_INC	65877

HEADING_BUG_DEC	65880

HEADING_BUG_INC	65879

HEADING_GYRO_SET	65568	This resets the gyro drift, aligning it to the mag compass

HUD_COLOR		 [*]

INCREASE_AUTOBRAKE_CONTROL	 [*]

INCREASE_CONCORDE_NOSE_VISOR	 [*]

INCREASE_CONCORDE_NOSE_VISOR_FULLY [*]

INCREASE_DECISION_HEIGHT		 [*]

INCREASE_HELO_GOV_BEEP		 [*]

INC_COWL_FLAPS		 [*]

INC_COWL_FLAPS1		 [*]

INC_COWL_FLAPS2		 [*]

INC_COWL_FLAPS3		 [*]

INC_COWL_FLAPS4		 [*]

JET_STARTER	65572	Now use TOGGLE_STARTERn (Note 1)

JOYSTICK_CALIBRATE	65573	? (not so much 'calibrate', more switch on or off?)

KNEEBOARD_VIEW		 [*]

KOHLSMAN_DEC	65884	Decrements altimeter pressure setting

KOHLSMAN_INC	65883	Increments altimeter pressure setting

LANDING_LIGHT_DOWN	65863

LANDING_LIGHT_HOME	65866	These landing light adjustments are only operative

LANDING_LIGHT_LEFT	65864	on the helicopter.

LANDING_LIGHT_RIGHT	65865

LANDING_LIGHT_UP	65862

LANDING_LIGHTS_OFF	66060

LANDING_LIGHTS_ON	66059

LANDING_LIGHTS_TOGGLE	65751

LANDME_TOGGLE	65581

LETTERBOX		 [*]

LOD_ZOOM_IN		 [*]

LOD_ZOOM_OUT		 [*]

MAGNETO	65586	Selects magneto. Use PLUS, MINUS to adjust

MAGNETO_BOTH	66026 	Operates on selected magnetos

MAGNETO_DECR	65634	Note: this was “MAGNETO_LEFT” before the Converter patch.

MAGNETO_INCR	65635	Note: this was “MAGNETO_RIGHT” before the Converter patch.

MAGNETO_LEFT	66025 	Sets mag to ‘left’.

MAGNETO_OFF	66023

MAGNETO_RIGHT	66024  	Sets mag to ‘left’.

MAGNETO_START	66027

MAGNETO1_BOTH	65930 	Individual magneto settings per engine. See also Note 1.

MAGNETO1_DECR	66126

MAGNETO1_INCR	66127

MAGNETO1_LEFT	65929

MAGNETO1_OFF	65927

MAGNETO1_RIGHT	65928

MAGNETO1_START	65931

MAGNETO2_BOTH	65936

MAGNETO2_DECR	66128

MAGNETO2_INCR	66129

MAGNETO2_LEFT	65935

MAGNETO2_OFF	65933

MAGNETO2_RIGHT	65934

MAGNETO2_START	65937

MAGNETO3_BOTH	65942

MAGNETO3_DECR	66130

MAGNETO3_INCR	66131

MAGNETO3_LEFT	65941

MAGNETO3_OFF	65939

MAGNETO3_RIGHT	65940

MAGNETO3_START	65943

MAGNETO4_BOTH	65948

MAGNETO4_DECR	66132

MAGNETO4_INCR	66133

MAGNETO4_LEFT	65947

MAGNETO4_OFF	65945

MAGNETO4_RIGHT	65946

MAGNETO4_START	65949

MAP_ZOOM_FINE_IN	65670

MAP_ZOOM_FINE_OUT	65673

MINUS	65548	MINUS is used to decrement a selected setting

MINUS_SHIFT	65756	MINUS_SHIFT is a finer decrement

MIXTURE_DECR	65777

MIXTURE_INCR	65775

MIXTURE_INCR_SMALL	65776

MIXTURE_LEAN	65778

MIXTURE_RICH	65774

MIXTURE1_DECR	65986

MIXTURE1_INCR	65984

MIXTURE1_INCR_SMALL	65985

MIXTURE1_LEAN	65987

MIXTURE1_RICH	65983

MIXTURE2_DECR	65991

MIXTURE2_INCR	65989

MIXTURE2_INCR_SMALL	65990

MIXTURE2_LEAN	65992

MIXTURE2_RICH	65988

MIXTURE3_DECR	65996

MIXTURE3_INCR	65994

MIXTURE3_INCR_SMALL	65995

MIXTURE3_LEAN	65997

MIXTURE3_RICH	65993

MIXTURE4_DECR	66001

MIXTURE4_INCR	65999

MIXTURE4_INCR_SMALL	66000

MIXTURE4_LEAN	66002

MIXTURE4_RICH	65998

MOUSE_AS_YOKE_OFF	65782

MOUSE_AS_YOKE_ON	65783

MOUSE_AS_YOKE_RESUME	65785

MOUSE_AS_YOKE_SUSPEND	65784

MOUSE_AS_YOKE_TOGGLE	65619

MP_ACTIVATE_CHAT	65905

MP_CHAT	65904

MP_GO_OBSERVER	65903

MP_PLAYER_CYCLE	65901

MP_PLAYER_SNAP	65902

MP_TRACK_TOGGLE	65900

NAV_RADIO	65585

NAV1_RADIO_FRACT_DEC	65642

NAV1_RADIO_FRACT_INC	65643

NAV1_RADIO_WHOLE_DEC	65640

NAV1_RADIO_WHOLE_INC	65641

NAV2_RADIO_FRACT_DEC	65646

NAV2_RADIO_FRACT_INC	65647

NAV2_RADIO_WHOLE_DEC	65644

NAV2_RADIO_WHOLE_INC	65645

NEW_MAP	65826

NEW_VIEW	65825

NEXT_VIEW	65827

NUMPAD1	65615	Apart from Axis TRIM, NUMPAD1 and NUMPAD7

NUMPAD2	65616	are the only elevator TRIM adjustment controls!

NUMPAD3	65617

NUMPAD4	65611

NUMPAD5	65612

NUMPAD6	65613

NUMPAD7	65607

NUMPAD8	65608

NUMPAD9	65609

PAN_DOWN	65735

PAN_LEFT	65671

PAN_LEFT_DOWN	65855

PAN_LEFT_UP	65854

PAN_RESET	65875

PAN_RIGHT	65672

PAN_RIGHT_DOWN	65857

PAN_RIGHT_UP	65856

PAN_TILT_LEFT	65873

PAN_TILT_RIGHT	65874

PAN_UP	65734

PANEL_1	65906

PANEL_2	65907

PANEL_3	65908

PANEL_4	65909

PANEL_5	65910

PANEL_6	65911

PANEL_7	65912

PANEL_8	65913

PANEL_9	65914

PANEL_LIGHTS _OFF	66057

PANEL_LIGHTS _ON	66056

PANEL_LIGHTS_TOGGLE	65750

PANEL_SELECT_1	65551

PANEL_SELECT_2	65747

PANEL_TOGGLE	65748

PARKING_BRAKES	65752

PAUSE_OFF	65795

PAUSE_ON	65794

PAUSE_TOGGLE	65561

PITOT_HEAT_OFF	66073

PITOT_HEAT_ON	66072

PITOT_HEAT_TOGGLE	65858

PLUS	65549	PLUS is used to increment a selected setting

PLUS_SHIFT	65757	PLUS_SHIFT is a finer increment

PREV_VIEW	65828

PROP_PITCH_DECR	65771

PROP_PITCH_HI	65772

PROP_PITCH_INCR	65769

PROP_PITCH_INCR_SMALL	65770

PROP_PITCH_LO	65768

PROP_PITCH1_DECR	66006

PROP_PITCH1_HI	66007

PROP_PITCH1_INCR	66004

PROP_PITCH1_INCR_SMALL	66005

PROP_PITCH1_LO	66003

PROP_PITCH2_DECR	66011

PROP_PITCH2_HI	66012

PROP_PITCH2_INCR	66009

PROP_PITCH2_INCR_SMALL	66010

PROP_PITCH2_LO	66008

PROP_PITCH3_DECR	66016

PROP_PITCH3_HI	66017

PROP_PITCH3_INCR	66014

PROP_PITCH3_INCR_SMALL	66015

PROP_PITCH3_LO	66013

PROP_PITCH4_DECR	66021

PROP_PITCH4_HI	66022

PROP_PITCH4_INCR	66019

PROP_PITCH4_INCR_SMALL	66020

PROP_PITCH4_LO	66018

RADIO_ADF_IDENT_DISABLE	65836

RADIO_ADF_IDENT_ENABLE	65841

RADIO_ADF_IDENT_TOGGLE	65846

RADIO_DME1_IDENT_DISABLE	65834

RADIO_DME1_IDENT_ENABLE	65839

RADIO_DME1_IDENT_TOGGLE	65844

RADIO_DME2_IDENT_DISABLE	65835

RADIO_DME2_IDENT_ENABLE	65840

RADIO_DME2_IDENT_TOGGLE	65845

RADIO_VOR1_IDENT_DISABLE	65832

RADIO_VOR1_IDENT_ENABLE	65837

RADIO_VOR1_IDENT_TOGGLE	65842

RADIO_VOR2_IDENT_DISABLE	65833

RADIO_VOR2_IDENT_ENABLE	65838

RADIO_VOR2_IDENT_TOGGLE	65843

READOUTS_FLIGHT	65737

READOUTS_SLEW	65736

RUDDER_LEFT	65688

RUDDER_CENTER	65689

RUDDER_RIGHT	65690

RUDDER_TRIM_LEFT		 [*]

RUDDER_TRIM_RIGHT		 [*]

SEE_OWN_AC_OFF	66120

SEE_OWN_AC_ON	66119

SEE_OWN_AC_TOGGLE	66118

SELECT_1	65538	Use SELECT_n after ENGINE to select engine(s)

SELECT_2	65539

SELECT_3	65540

SELECT_4	65541

SET_FUEL_TRANSFER_AFT		 [*]

SET_FUEL_TRANSFER_AUTO		 [*]

SET_FUEL_TRANSFER_FORWARD		 [*]

SET_FUEL_TRANSFER_OFF		 [*]

SIM_RATE	65555

SIM_RATE_DECR	65790

SIM_RATE_INCR	65789

SITUATION_RESET	65591

SITUATION_SAVE	65575

SLEW_AHEAD_MINUS	65745

SLEW_AHEAD_PLUS	65739

SLEW_ALTIT_DN_FAST	65624

SLEW_ALTIT_DN_SLOW	65623

SLEW_ALTIT_FREEZE	65622

SLEW_ALTIT_MINUS	65626

SLEW_ALTIT_PLUS	65625

SLEW_ALTIT_UP_FAST	65620

SLEW_ALTIT_UP_SLOW	65621

SLEW_BANK_MINUS	65738

SLEW_BANK_PLUS	65740

SLEW_FREEZE	65742

SLEW_HEADING_MINUS	65744

SLEW_HEADING_PLUS	65746

SLEW_LEFT	65741

SLEW_OFF	65730

SLEW_ON	65731

SLEW_PITCH_DN_FAST	65627

SLEW_PITCH_DN_SLOW	65628

SLEW_PITCH_FREEZE	65629

SLEW_PITCH_MINUS	65633

SLEW_PITCH_PLUS	65632

SLEW_PITCH_UP_FAST	65631

SLEW_PITCH_UP_SLOW	65630

SLEW_RESET	65796

SLEW_RIGHT	65743

SLEW_TOGGLE	65557

SMOKE_OFF	66050

SMOKE_ON	66049

SMOKE_TOGGLE	65559

SOUND _OFF	66063

SOUND _ON	66062

SOUND_TOGGLE	65552

SPOILERS_ARM_OFF	66067

SPOILERS_ARM_ON	66066

SPOILERS_ARM_TOGGLE	65853

SPOILERS _OFF	66065

SPOILERS _ON	66064

SPOILERS_TOGGLE	65589

STROBES_OFF	66053

STROBES_ON	66052

STROBES_TOGGLE	65560

SYNC_FLIGHT_DIRECTOR_PITCH		 [*]

THROTTLE_10		 [*]

THROTTLE_20		 [*]

THROTTLE_30		 [*]

THROTTLE_40		 [*]

THROTTLE_50		 [*]

THROTTLE_60		 [*]

THROTTLE_70		 [*]

THROTTLE_80		 [*]

THROTTLE_90		 [*]

THROTTLE_CUT	65604

THROTTLE_DECR	65602

THROTTLE_FULL	65596

THROTTLE_INCR	65598

THROTTLE_INCR_SMALL	65600

THROTTLE1_CUT	65967

THROTTLE1_DECR	65966

THROTTLE1_FULL	65963

THROTTLE1_INCR	65964

THROTTLE1_INCR_SMALL	65965

THROTTLE2_CUT	65972

THROTTLE2_DECR	65971

THROTTLE2_FULL	65968

THROTTLE2_INCR	65969

THROTTLE2_INCR_SMALL	65970

THROTTLE3_CUT	65977

THROTTLE3_DECR	65976

THROTTLE3_FULL	65973

THROTTLE3_INCR	65974

THROTTLE3_INCR_SMALL	65975

THROTTLE4_CUT	65982

THROTTLE4_DECR	65981

THROTTLE4_FULL	65978

THROTTLE4_INCR	65979

THROTTLE4_INCR_SMALL	65980

TOGGLE_AFTERBURNER		 [*]

TOGGLE_AFTERBURNER1		 [*]

TOGGLE_AFTERBURNER2		 [*]

TOGGLE_AFTERBURNER3		 [*]

TOGGLE_AFTERBURNER4		 [*]

TOGGLE_ALL_STARTERS		 [*]	Use instead of JET_STARTER for all engines? (Note 1)

TOGGLE_ALTERNATE_STATIC		 [*]

TOGGLE_ALTERNATOR1		 [*]

TOGGLE_ALTERNATOR2		 [*]

TOGGLE_ALTERNATOR3		 [*]

TOGGLE_ALTERNATOR4		 [*]

TOGGLE_AUTOFEATHER_ARM		 [*]

TOGGLE_AVIONICS_MASTER		 [*]

TOGGLE_BEACON_LIGHTS		 [*]

TOGGLE_DME		 [*]

TOGGLE_ELECTRICAL_FAILURE		 [*]

TOGGLE_ELECTRIC_VACUUM_PUMP	 [*]

TOGGLE_ELECT_FUEL_PUMP		 [*]

TOGGLE_ELECT_FUEL_PUMP1		 [*]

TOGGLE_ELECT_FUEL_PUMP2		 [*]

TOGGLE_ELECT_FUEL_PUMP3		 [*]

TOGGLE_ELECT_FUEL_PUMP4		 [*]

TOGGLE_ENGINE1_FAILURE		 [*]

TOGGLE_ENGINE2_FAILURE		 [*]

TOGGLE_ENGINE3_FAILURE		 [*]

TOGGLE_ENGINE4_FAILURE		 [*]

TOGGLE_FLIGHT_DIRECTOR		 [*]

TOGGLE_GPS_DRIVES_NAV1		 [*]

TOGGLE_HYDRAULIC_FAILURE		 [*]

TOGGLE_LEFT_BRAKE_FAILURE		 [*]

TOGGLE_LOGO_LIGHTS		 [*]

TOGGLE_MASTER_ALTERNATOR		 [*]

TOGGLE_MASTER_BATTERY		 [*]

TOGGLE_MASTER_BATTERY_ALTERNATOR [*]

TOGGLE_NAV_LIGHTS		 [*]

TOGGLE_PITOT_BLOCKAGE		 [*]

TOGGLE_PRIMER		 [*]

TOGGLE_PRIMER1		 [*]

TOGGLE_PRIMER2		 [*]

TOGGLE_PRIMER3		 [*]

TOGGLE_PRIMER4		 [*]

TOGGLE_PROPELLER_DEICE		 [*]

TOGGLE_PROPELLER_SYNC		 [*]

TOGGLE_RADIO_COMM1_FREQUENCY	 [*]

TOGGLE_RADIO_COMM2_FREQUENCY	 [*]

TOGGLE_RADIO_COMMNAV1_TEST	 [*]

TOGGLE_RADIO_COMMNAV2_TEST	 [*]

TOGGLE_RADIO_NAV1_FREQUENCY	 [*]

TOGGLE_RADIO_NAV2_FREQUENCY	 [*]

TOGGLE_RAD_INS_SWITCH		 [*]

TOGGLE_RECOGNITION_LIGHTS		 [*]

TOGGLE_RIGHT_BRAKE_FAILURE		 [*]

TOGGLE_STARTER1		 [*]	Use instead of JET_STARTER  (Note 1)

TOGGLE_STARTER2		 [*]

TOGGLE_STARTER3		 [*]

TOGGLE_STARTER4		 [*]

TOGGLE_STATIC_PORT_BLOCKAGE	 [*]

TOGGLE_STRUCTURAL_DEICE		 [*]

TOGGLE_TAXI_LIGHTS		 [*]

TOGGLE_TOTAL_BRAKE_FAILURE		 [*]

TOGGLE_VACUUM_FAILURE		 [*]

TOGGLE_WING_LIGHTS		 [*]

TRUE_AIRSPEED_CAL_DEC		 [*]

TRUE_AIRSPEED_CAL_INC		 [*]

TRUE_AIRSPEED_CAL_SET		 [*]

TRUE_AIRSPEED_CAL_DEC	65886	Adjusts small sub-dial in Cessna's ASI

TRUE_AIRSPEED_CAL_INC	65885

VIEW	65606

VIEW_AUX_00		 [*]

VIEW_AUX_01		 [*]

VIEW_AUX_02		 [*]

VIEW_AUX_03		 [*]

VIEW_AUX_04		 [*]

VIEW_AUX_05		 [*]

VIEW_DOWN	65682

VIEW_FORWARD	65674

VIEW_FORWARD_LEFT	65681

VIEW_FORWARD_RIGHT	65675

VIEW_LEFT	65680

VIEW_MODE	65567

VIEW_MODE_REV	65749

VIEW_REAR	65678

VIEW_REAR_LEFT	65679

VIEW_REAR_RIGHT	65677

VIEW_RESET		 [*]

VIEW_RIGHT	65676

VIEW_TYPE	65829

VIEW_TYPE_REV	65830

VIEW_WINDOW_TO_FRONT	65576

VOR_OBS	65583

VOR1_OBI_DEC	65662

VOR1_OBI_INC	65663

VOR2_OBI_DEC	65664

VOR2_OBI_INC	65665

XPNDR	65556

XPNDR_1_INC	65654

XPNDR_10_INC	65653

XPNDR_100_INC	65652

XPNDR_1000_INC	65651

YAW_DAMPER_OFF	66070

YAW_DAMPER_ON	66069

YAW_DAMPER_TOGGLE	65793

ZOOM_1X	65550

ZOOM_IN	65655

ZOOM_IN_FINE	65754

ZOOM_MINUS	65718

ZOOM_OUT	65656

ZOOM_OUT_FINE	65755

ZOOM_PLUS	65719

Note 1: In FS98, and possibly also FS2K (at least for transposed FS98 aircraft) the MAGNETOn_ controls also operate the JET_STARTER for the nth Engine. This is the correspondence:

MAGNETOn_OFF	sets Jet Starter n to OFF

MAGNETOn_RIGHT	sets Jet Starter n to START

MAGNETOn_LEFT	sets Jet Starter n to GEN

Other values also set Jet Starter n to GEN. The MAGNETOn_SET analogue input also controls the Jet Starter in a similar way. Remember that to start a jet you need fuel switched on for that engine (by "MIXTUREn_RICH" for instance), and of course appropriate selection of fuel tanks by FUEL_SELECTOR_ controls.

In FS2K, at least for FS2K aircraft with the nice new startup sequence, the best way to start the engines appears to be to use the TOGGLE_STARTERn controls for each engine (or, less realistically, the TOGGLE_ALL_STARTERS control). These need to be continuously operated until the jets fire—you can hear when this happens. The BUTTON_DOWN_REPEAT facility can be used for this when the starter is assigned to a joystick button.